Image Processing

Kirk Martínez

Birkbeck/University College, London

What is Image Processing?

Image processing is the manipulation of computerised images of real objects. There are many fields in the subject ranging from picture coding for television, object recognition for robots, image restoration, document, satellite and medical image processing. Throughout this diverse subject there are common tools and fundamentals based on the mathematics of signal processing. There is usually a clear distinction between image processing and computer graphics, which is the generation of synthetic images, although the two can merge into image computing.

All television signals go through a process to compress them into less data and make effective use of telecommunication channels. The higher definition TV systems being released now depend heavily on digital processing of the images. Moving picture coding has the extra dimension of motion, which can help or hinder. For example compression can make use of the fact that little changes from frame to frame, but there is so much data in one second that processing must be extremely fast: too fast for most computers!

Robot vision systems allow machines with cameras attached to automatically inspect objects on conveyor belts for example, spotting cracked bottles, odd-shaped mouldings etc. Systems to allow freely moving robots to find there way around have been developed but are still fairly primitive (not technologically!). We have a high expectation of robot vision because we have such a good visual processor in our heads, and years of training to use it. Computer vision still has a hard time trying to isolate the objects in a scene, before even trying to categorise them. Typical systems may use two cameras to give stereo vision and may split up the scene into boundaries and edges. This is something we know human vision does, so it seems a good idea to copy it.

Image restoration has been extensively developed by NASA which uses many techniques to improve the images it gets from space probes and satellites. One of the most spectacular is the de-blurring of images making use of known conditions such as motion. In remote sensing, satellite images which take many images through the spectrum can be processed to characterise crops or soil types for example. Pairs of satellite images can be processed to give a height estimate of the scene, so that maps and computer graphics can be made to represent it.

Document image processing has developed to aid in the management of paper-less filing systems. Document scanners use many techniques to process the image of a page, before applying character recognition to read the actual letters and convert them into computer text rather than just dots (this is also known as Optical character recognition: OCR). FAX machines use a picture compression scheme to take advantage of the fact that most of the page is blank.

All these techniques can be applied to images of art, although wether some will work has yet to be discovered. Paintings for example may contain shading and colouring which are acceptable for human recognition, but far from the real image. However techniques based on "training" may be better for recognition of objects in paintings.

How are digital images made?

A digital image starts its life as an analogue signal of continuous levels and must be sampled by an analogue to digital (A/D) converter. This provides a grid of samples (pels: picture elements) represented as numbers, which may correspond to the lightness of the image. Typically one byte is used per pel, so levels from 0 to 255 are stored. If the image is quantised to fewer than around 64 levels (ie 6 bits) then false lines or contours may be seen.

Television signals are extremely fast (over 500 lines in one frame in 1/25th of a second or one pel every 0.00000001s) so a framestore is used which can quickly store the incoming data in memory (RAM). This process is popularly known as grabbing and images are scanned top down, left to right. The computer can then read the data at its slower speed to store/process it.

0. Analogue (top) to digital conversion

Desktop scanners usually read lines of the image much more slowly, and the computer can read them directly. To make a colour image three images must be taken through red/green/blue filters to mimic the eye. These RGB values can then be sent to the corresponding beams in a colour monitor to produce a colour image. In practice some calibration is needed to make the colour look right, and some colours are impossible to reproduce. Once images are digitised they can be stored on disks, tapes etc permanently and will not change unless the storage medium is damaged or ages. This can be prevented by copying as often as the medium necessitates. As the images are represented as computer numbers they can easily be processed.

Resolution

An image with too few samples looks blocky because the pels are too big. A television picture or camera image is usually stored as 512x512 pels, which is often more than enough for such sources. High resolution CCD cameras can give 6000x4000 by slowly scanning a single line sensor. A 35mm slide scanner usually provides around 4000x4000 pels, which shows the high resolution of photography. If insufficient resolution is used, then fine detail is lost and may even cause spurious patterns or aliasing. Classical examples of this are the bands produced by overlapping net curtains due to one pattern sampling another by occlusion. Wagon wheels which turn the wrong way on films are aliasing in time, due to the film frames being taken too slowly and catching the wheel at various stages of rotation. Television uses 25 frames per second, which is just unnoticeable by the eye (but try looking at a TV with your peripheral vision, which is more sensitive to motion). Modern displays refresh the screen at 70 frames per second.

Contrast enhancements

One of the simplest image operations is thresholding, where values below a certain level are set to black and the rest are set to white. This is often used to simplify the image to cut out areas or for further recognition. Level slicing is similar but levels between two values become black, with others turning white. This is useful for images where ranges of values are meaningful, for example an image representing temperature or the output of another operation. False colour can show many ranges as different colours, typically low values are shown as blue, with colours of the rainbow assigned to larger value ranges. These give more information and are common in satellite imaging.

§0. 128x128 image: Campin

Monitors, cameras and the eye have different responses to their inputs. For example a monitor will produce a brighter step between inputs 0.8-0.9V than from 0.2-0.3V. If a bright scene has a part which emits twice as much light, this will seem to the eye as less different in brightness than a dark scene with an area which emits twice the amount of light. The relation for each of these cases can be expressed as a power law:

response = input gamma

and can easily be applied to images. A gamma greater than one tends to expand the dark regions and compress the lighter regions. Typical displays have a gamma of 2-3, whereas the eye has a gamma of 0.3-0.5 so they can just cancel each other out (ie: monitor has a square law, eye has square root). In practice some gamma correction is needed to make sure that equal steps of voltage to the monitor give us equal visual steps of brightness, so that the tonal balance is correct.

The histogram of an image, which counts the number of pels of each value, can be used in many ways. Good threshold values can be found by looking for natural groups of values for example. A simple contrast stretch to spread the values over the whole 0-255 range is useful but using the histogram to spread the values out even more produces dramatic effects. This is known as histogram equalisation which attempts to make the histogram of the new image uniform. Figure 4 shows a portion of an X-ray image, which shows underdrawing. Figure 5 shows the effect of histogram equalisation. Figures 6 and 7 below show their corresponding histograms. It is also possible to force the image to have a histogram close to one given, by redistributing the values.

§0. Campin thresholded at average

0. Gamma = 2 (ie square)

0. gamma = 0.5 (square root)

0. Dujardin Xray

0. histogram equalisation

0. Histogram of Xray

0. histogram equalised

Filtering

Taking a weighted sum of the local area around each pel is a convenient way of carrying out many operations. Consider a local region of 9 pels around the first pel of an image. These pels can be averaged and the value placed in a new image. This operation can be represented by the blur "mask" below (ie add each neighbour and divide by 9). If a local average is made for each pel in the image and all the averages are stored in a new image, then a blurred image is made. This is illustrated below for one pel, where the slightly brighter value 15 is reduced to the local average of 10. It is assumed that the result is divided by 9 (sum of mask).

| 1 | 1 | 1 |  | 10 | 10 | 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | on | 5 | 15 | 10 | = |  | 10 |  |
| 1 | 1 | 1 |  | 10 | 10 | 10 |  |  |  |  |
| Blur mask |  |  |  |  |  | Some pels |  |  |  |  |  | New pel |  |  |  |

Thus for this filter small variations in the region are averaged out: ie high frequencies are cut out but low frequencies left, which is why it blurs (also called a low-pass filter). This process of applying a mask is known as convolution.

| -1 |  |  | -1 | -1 |  | 1 |  |  | 1 | 1 |  | -1 |  | -1 | -1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -1 | 8 | -1 |  | -2 | -2 | -2 |  | -1 | 10 | -1 |
| -1 | -1 | -1 |  | 1 | 1 | 1 |  | -1 | -1 | -1 |
| a) High pass |  |  |  |  |  | b) Line detector |  |  |  |  |  | c) sharpen |  |  |  |

The high pass mask shown in a) means that the surrounding pels are summed, then subtracted from 8 times the centre. In a smooth picture area this would give zero, where the centre is darker or lighter it gives a large value (45 with the example above). Thus it senses where small sized changes are occurring (ie high frequencies), but not smooth areas: hence high pass. The second filter c) would give zero for smooth areas, but highest values for dark vertical lines of width one pel. Mask c) is really the high pass filter a) with extra weighting to the centre. This can be seen as the high pass image added to two parts of the original, which produces a sharpening effect. By combining filters like b) with different orientations specific features can be detected. Larger masks detect or affect larger features. So a 10x10 blur mask would severely blur the image. Masks which make negative values such as the three above are often scaled down and shifted up by 128 to make all the values visible.

Convolution filtering is a fundamental tool in image processing and is mathematically understood to the extent that filters can be designed to cut/boost specific image frequencies. By adding a fraction of the result of filter a) to the original image, a sharper appearance can be made. Filters can be used as edge detectors which give simplified information for subsequent analysis or recognition.

0. Blurred version

0. High pass version

0. Line detector

0. Sharpen filter

The frequency world

Like sound can be split into its component frequencies, so can images. Image frequencies are related to how fast the values are changing across the image, so a picture of graph paper contains specific high frequencies related to the spacing of the lines and their thickness. An image of a pool of custard, or a subtly shaded wall would contain mainly low frequencies. In images the frequencies have directions, so graph paper has two main directions. If you think of waves in the sea, large waves have small waves superimposed on them, which may go in different directions. It can be imagined that the many shapes of waves can b e made by superimposing many simple waves. The imaginary waves in a still image do not move, but an image can be made with a set of waves of brightness with certain frequencies and amplitudes.

An image can be split into its frequencies by a Fourier transform, which gives data which is hard to visualise but easy to process. The power spectrum shows the relative amounts of each frequency, so it is a convenient way to look at the data, with low frequencies in the centre and higher ones towards the outside. Figure 14 shows this for the Campin image, with brightness indicating strength. Note that images contain mostly low frequencies. The Fourier transform is reversible, so the frequencies can be recombined back into an image. This means it is possible to remove or boost some frequencies and hence filter the image. Blurring the image simply involves reducing the high frequency components for example. Often a frequency filter will be made in the form of a frequency image, which is multiplied by the image's frequency image. If an image contains a repetitive pattern it is often possible to isolate it in the frequency domain and remove it. Advanced versions of this technique have been used to remove the canvas texture from x-ray images in order to compare them to visual images better.

0. Power Spectrum of Campin

The convolution filter masks mentioned earlier have a precise interrelationship with their frequency versions. Some filters can be designed in their frequency form then converted into convolution masks and visa versa. The frequency domain can also be used to classify image information as different textures have particular frequency patterns for example.

Moving images can also be considered in the frequency domain. A uniform white image which fades up and down in brightness would have a particular temporal frequency but hardly any spatial frequencies. The three dimensional frequency space of television pictures is of interest to broadcasters as it can be used to compress the information and study defects.

Sampling and aliasing

Samples should be taken at twice the maximum frequency the data contains, to give at least two samples for a cycle (this is called Nyquist sampling). So to record an image of an object which has 0.1mm cracks, the samples should represent at most 0.05mm of object. In practice more will be needed to clearly see the crack and compensate for blurring etc. When a high resolution image is made, some way of viewing the whole image is needed due to limitations in displays (around 2000x2000). This involves simple sampling of the image but taking one in six pels for example is not good enough, as it can cause distortions known as aliasing where the high frequencies cause errors. Ideally all frequencies below half the sampling frequency should be removed first. This can be simplified to filtering with a suitable low-pass filter such as a block average, of the appropriate size. To shrink a picture by 2:1, 2x2 blocks can be averaged every two pels. This is not the ideal filter but is generally used for simplicity. Low pass filtering has to be carried out carefully because if the filter is too good and cuts out frequencies completely at one frequency (a "brick wall" filter!) some other frequency components can suddenly appear as ripples or ringing. In practice a smoother cut-off point is use to prevent this.

Image data compression

A digital image starts out with much more data then an analogue one because each sample contains many bits, so to transmit or store it requires more bandwidth or space. Digital images also contain redundant data such as large smooth areas, especially in documents. Picture compression uses many techniques based on removing the redundancies and exploiting the properties of the eye. A lossless technique where no actual degradation occurs can only achieve around 3:1 compression with non-document images, but is ideal for archiving where no loss can be tolerated. Documents with large uniform areas can be compressed such as run-length coding where the number of consecutive identical values are stored instead.

One fundamental property of images is that each pel is usually similar to its neighbours, so it is usually possible to predict what it will be from previous pels. The difference between the real value and the prediction is usually very small and its statistics are so peaked that compression is made easier. Figure 15 shows a typical histogram of the difference between the previous and current pel (the centre is zero). Statistical coding can assign short codes with perhaps two bits, to the common values such as 0 and 1 and longer codes to the unlikely values. By storing the large differences less accurately it is possible to use even fewer bits. This corrupts the image but only where there are big steps, where the eye can not see errors anyway. This technique (DPCM: Differential Pulse Code Modulation) can achieve a compression of around 8:1 with little visual effect and is commonly used in broadcasting.

§0. Previous pel difference histogram for Campin image

Transform coding first transforms the image into another space, usually similar to the frequency domain, before removing redundancies and coarsely quantizing some elements. So for example an image may be split into 8x8 pel blocks, each is coded separately by transforming into an 8x8 frequency space, then more bits are assigned to some frequencies than others. This is possible since the eye can not easily see errors in the high frequencies for example. For more compression, the higher frequencies can simply be omitted. A statistical technique is then used to compress these codes even more. On decoding the image from the frequency strengths some errors can occur, usually blurring or block effects. A similar technique known as DCT (Discrete Cosine Transform) is used in the JPEG (Joint Photographic Experts Group) compression scheme which is becoming a standard for image compression. It has been implemented in hardware (it is extremely slow on a normal processor) and can compress images to 24:1 with little visual effect.

Image Storage

Images are usually stored as separate files with a header giving information such as the resolution, type etc. A typical video resolution image of around 512x512 pels, with one byte for each of RGB occupies 768k bytes (1k=1024). This will fit onto most floppy disks easily and hard disks will store many of these. For example a 1024Mb disk (1Mb = million bytes) could hold around 1300 of these images. Images containing all the information on a 35mm slide are typically around 4000x3000 pels or 36Mb, so very few can be stored on one hard disk. The problems with data from 10x8" transparencies are even worse. Currently one viable way to have many of these images on line is to use optical disks. CDROM disks hold around 600Mb but are slow compared to 5.25" optical disks, which store about the same (up to 1Gb) and 12" disks can hold 6Gb each. By placing many inside a "juke-box" which can automatically select disks huge amounts can be stored. By using data compression, many more images can be stored and because there is less on the disk it is faster to read, as long as the decompression is fast enough.

Conclusions

Image processing used to be very expensive but can now be carried out on personal computers with commercially available software. There are many techniques, described in papers and books on the subject: this paper can only cover some fundamentals. There are a growing number of researchers using and developing image processing in museums, where images are a fundamental part of life and image processing can be a valuable tool.

Bibliography

Pearson, D.E. (Ed.), Image Processing, McGraw-Hill, 1991. ISBN 0-07-707323-1

Gonzalez R.C. and Wintz P., Digital Image Processing, 2nd edition, Addison-Wesley, Reading, Mass. 1987.

Young T.Y. and Fu K.S., Handbook of Pattern Recognition and Image Processing, Academic Press, Orlando, FL. 1986.

Ballard D.H. and Brown C.M., Computer Vision, Prentice Hall, Englewood Cliffs, NJ. 1982.

Pratt W.K., Digital Image processing, Wiley, 1978.

Pearson D.E., Transmission and Display of Pictorial Information, Pentech Press, London, 1975.

Foley J.D., van Dam A., Feiner S.K. and Hughes J.F., Computer Graphics: Principles and Practice, 2nd edition, Addison-Wesley, Reading Mass. 1990.